# SSL VPN Configuration Documentation

## 3. HQ FortiGate SSL VPN Configuration

### Step 1: SSL VPN Settings

Navigate to VPN → SSL-VPN Settings

Configuration:

- Listen on Interface: port2 (LAN)

- Listen on Port: 10443 (HTTPS)

- Server Certificate: Fortinet_Factory

- Idle Timeout: 3000 seconds

- Tunnel Mode IP Pools: SSLVPN_TUNNEL_ADDR1 (10.212.134.200 - 10.212.134.210)

- IPv6 Pools: SSLVPN_TUNNEL_IPv6_ADDR1 (fdff:ffff::/120)

- Default Portal: full-access

Portal Settings (full-access):

- Tunnel Mode: Enabled

- IPv6 Tunnel Mode: Enabled

- Web Mode: Enabled

- IP Pools: SSLVPN_TUNNEL_ADDR1

- IPv6 Pools: SSLVPN_TUNNEL_IPv6_ADDR1

### Step 2: Create IP Pool for SSL VPN Users

Navigate to Policy & Objects → Addresses

Create New IP Pool:

- Name: SSLVPN_TUNNEL_ADDR1

- Type: IP Range

- Start IP: 10.212.134.200

- End IP: 10.212.134.210

### Step 3: Create User Account

Navigate to User & Authentication → User Definition

Create New User:

- Username: vpnuser

- Type: Local User

- Password: (Set secure password)

- Status: Enabled

### Step 4: Create User Group

Navigate to User & Authentication → User Groups

Create New Group:

- Name: SSL_VPN_USERS

- Type: Firewall

- Members: vpnuser

### Step 5: Firewall Policy Configuration

SSL VPN Access Policy

Navigate to Policy & Objects → Firewall Policy

Policy Details:

- Name: SSL_VPN_Access

- Incoming Interface: SSL_VPN tunnel interface (ssl.root)

- Outgoing Interface: LAN (port2)

- Source: SSLVPN_TUNNEL_ADDR1

- Destination: all

- Schedule: always

- Service: ALL

- Action: ACCEPT

- NAT: Enabled (Use Outgoing Interface Address)

- Security Profiles: AntiVirus, Web Filter enabled

### Step 6: Web-Based Mode Testing

Test 5.1: SSL VPN Portal Access Test

Verify web portal accessibility and authentication

- SSL-VPN portal accessible at https://192.168.32.135:10443 or <ip:port>

- Portal shows: "The SSL-VPN portal has been enabled for tunnel mode use only"

- FortiClient launch and download options available

- Connection history visible with previous sessions

Test 5.2: Web Portal Services Test

Test web-accessible services through portal

- Access to the VPN target

- Then login as a vpnuser

- View the details

- Verify login from the fortigate firewall GUI (Dashboard → Network)

- See the details to view that its web mode access on the SSL VPN

### Step 7: FortiGate SSL VPN tunnel mode test

FortiClient Connection configuration

VPN Connection Settings

Connection Parameters:

- VPN Name:

- Connection Type: SSL-VPN

- Remote Gateway: https://<IP>:10443

- Custom Port: 10443

- Authentication: Username/Password

- Username: vpnuser

- Single Sign-On: Disabled

- Client Certificate: None

- Dual-stack IPv4/IPv6: Enabled

### Step 8: connection and monitoring

In this step, after establishing the connection using tunnel mode, I selected Forti-Lab and tested the setup using the VPN user I had previously created.
As shown in the image, the monitoring interface displays several key details for each connected user:

- Username – identifies the authenticated VPN user.

- IP Address – shows the assigned IP for the VPN session.

- Connection Duration – indicates how long the user has been connected.

- Bytes Sent and Received – displays the amount of data transmitted during the session.

### Step 9: Monitoring and Troubleshooting

Active Connections Monitoring

SSL-VPN Dashboard Shows:

- Active Users: 1

- Connection Mode: Web

- Username: vpnuser

- Remote Host: 192.168.1.9

- Tunnel Ip: 10.212.134.200

- Duration: 44s

- Source interface: Wan1

- Tunnel IP: Assigned from SSLVPN_TUNNEL_ADDR1 pool

Document Version: 1.0
Last Updated: October 28, 2025
FortiGate Version: 7.2.12
Tested With: FortiClient 7.2.12