Week 3

SD-WAN Implementation Documentation

Introduction & Purpose of SD-WAN

SD-WAN (Software-Defined Wide Area Network) is an advanced solution for managing WAN networks that provides:

- The ability to combine multiple internet connections (WAN links) for smart traffic distribution.

- Quality-based routing to optimize traffic depending on the service type (e.g., VoIP, Video, Web).

- Automatic failover if one link goes down, ensuring uninterrupted user experience.

- Load balancing to utilize all available links efficiently.

- Continuous performance monitoring (latency, jitter, packet loss) to improve network reliability.

In short: SD-WAN makes the network more intelligent, flexible, and reliable compared to a single fixed WAN link.

Project Environment

- Number of Internet Links: 2 WAN links

- Local Network: 10.10.10.0/24

- Device: FortiGate Firewall

- Goal: Route internal traffic to the internet efficiently and manage traffic based on service type (e.g., VoIP calls, YouTube streaming).

Implementation Steps

Step 1: Verify Initial Configuration

- Checked Firewall Policies: No policies are active.

- Checked Routing: No static routes or default routes exist.

- Purpose: Ensure a clean environment before enabling SD-WAN.

Step 2: Add Internet Links as SD-WAN Members

- Each WAN link is added as a Member inside SD-WAN:

  - WAN1 → Member 1

  - WAN2 → Member 2

- Purpose: Integrate all WAN links under a single SD-WAN zone for centralized management.

Step 3: Create SD-WAN Zone

- Created a SD-WAN Zone named SD-WAN-Zone.

- Added all WAN members (WAN1, WAN2) into this zone.

Step 4: Configure Default Static Route

- Static Route Configuration:

  - Destination: 0.0.0.0/0

  - Interface: SD-WAN-Zone

- Purpose: Direct all outbound traffic through the SD-WAN zone instead of individual WAN interfaces.

Step 5: Configure Firewall Policy

- Created a firewall policy named Internet Access:

  - Incoming Interface: LAN

  - Outgoing Interface: SD-WAN-Zone

  - Source: Local Subnet 10.10.10.0/24

  - Destination: All

  - Schedule: All

  - Service: All

- Purpose: Ensure internal traffic exits to the internet via the SD-WAN zone.

Step 6: Configure Performance SLA

- Name: Internet_Link_Check

- Probe Mode: Prefer Passive

- Protocol: Ping

- Servers: 8.8.8.8, 4.4.2.2

- Participated Members: All SD-WAN Members

- SLA Targets:

  - Latency: 200ms

  - Jitter: 50ms

  - Packet Loss: 5%

- Purpose: Monitor link quality and enable intelligent routing decisions based on performance.

Step 7: Create SD-WAN Rules

- Example rules implemented:

  - Traffic from subnet 10.10.10.0/24 to YouTube → routed via WAN1

  - Traffic from subnet 10.10.10.0/24 for VoIP calls → routed via the best performing link automatically

- Purpose: Direct traffic efficiently based on application type and link quality.

Step 8: Load Balancing Configuration

- Mode: Source-IP based

- Purpose: Distribute traffic across WAN links evenly while keeping sessions consistent per source IP.

- Configured in GUI (or optionally CLI for mode selection).

Step 9: Testing & Monitoring

- Observed SD-WAN member status:

  - WAN1: Up

  - WAN2: Down

- Tested traffic routing for YouTube and VoIP services to verify SLA rules and best-quality routing.

- Verified failover functionality by simulating WAN link failure.

- Monitored latency, jitter, and packet loss via SD-WAN Monitor dashboard.

Conclusion

The SD-WAN implementation provides:

- Intelligent routing and application-aware traffic steering.

- Automatic failover for uninterrupted connectivity.

- Efficient utilization of all WAN links with load balancing.

- Real-time performance monitoring for proactive network management.

Mohab Nasser